- 1 UV ozone concentration analyzer OZONE-500

Product Introduction

The OZONE-500 ozone concentration analyzer is based on the principle of 254 nm ultraviolet light absorption method and designed according to the Beer Lambert law. The instrument complies with the HJ590-2010 "Ozone Calibration - Ultraviolet Method" and GB/T37397-2019 "Ozone Calibration Analyzer" standards. The instrument adopts imported ultraviolet electronic light source, built-in sampling air pump, and is equipped with pressure and temperature compensation and display, real-time flow monitoring, automatic fault diagnosis and other functions. The integrated aluminum alloy body design, combined with a damping rubber handle and engineering protection box, makes it easy to carry.

Product Image

Features

Adopting the standard 254nm ultraviolet absorption detection principle;

Portable, with ppb level detection accuracy;

30 second high-speed startup;

Dew Balance dehumidification technology;

Export data from USB flash drive in PDF, CSV, TXT formats;

Standard lithium battery, capable of continuous operation for 6 hours;

WiFi, USB and other signal outputs;

Bluetooth printer.

Product parameters

Measurement range: 0-1ppm, 0-100ppm, 0-500ppm

Accuracy: ± 2% (FS)

Resolution: 0.001ppm

Measurement time: 3 seconds

Signal output: USB, WiFi, RS485, 0-10V, 4-20mA

Printing method: External Bluetooth printer

Power on preheating: 30 seconds

Storage data: 1 million sets

Usage environment: Temperature 0-50 ℃, humidity 0-85% RH (no dew)

Sampling flow rate: 1L/min, real-time flow detection

Display screen: 5-inch touch screen display

Light source lifespan: 15000h

Power supply: DC16.8V/3A

Dimensions: 290 × 220 × 120mm (length × width × height)

Weight: 3.8kg