ND-16/ND-16A neutron personal dosimeter

Product introduction

ND-16/ND-16A neutron personal dosimeter is mainly used to measure the personal dose equivalent caused by neutron gamma and X-ray radiation, and is suitable for dose equivalent measurement in radioactive workplaces. The dosimeter is controlled by a low-power single-chip microcomputer, which realizes the functions of LCD drive display, measurement data storage, related parameter setting, detector fault alarm, battery voltage detection, over-threshold acousto-optic alarm, charging and so on.

This dosimeter can be used in radiation medical treatment, industrial nondestructive testing, radioisotope application, occupational disease prevention, radiation dose equivalent measurement in troops, nuclear power plants and the periphery of radioactive workplaces.

Technical indexes

|  | ND-16 | ND-16A |
| --- | --- | --- |
| Detector type | - 6Li coated compound semiconductor detector | 6Li coated compound semiconductor detector+energy compensated GM compound detector |
| Detection radiation type | Neutron ray | Neutrons, gamma rays and x-rays |
| cumulative dose | 1.0μSv～9999mSv | - X/γ：1.0μSv～9999mSv； - Neutron: 1.0 μ SV ~ 9999 msv |
| dosage rate | 10μSv/h～100mSv/h | - X/γ：0.10μSv/h～100mSv/h； Neutron: 10μ SV/h ~ 100mSV/h |
| Energy response range | 0.025eV～14MeV | - X/γ：50keV～3MeV； - Neutron: 0.025eV～14MeV ~ 14 mev |
| Relative inherent error | Hp(10) is better than 20% |  |
| Alarm function | It has alarm functions such as dose rate, cumulative dose and working time, and the alarm threshold can be continuously adjusted. The alarm methods are acousto-optic alarm and LCD alarm. |  |
| Power Supply | Built-in lithium battery can run continuously for more than 500 h after one charge in the background mode. |  |
| Working temperature and humidity | -10℃ ~ 45℃, ≤90%RH (no condensation) |  |
| Appearance size | - Length 95mm* width 60mm* thickness 27.5mm (excluding back clip) |  |
| weight | About 190g |  |